CS 20 Laboratory 1: Introduction to Electric Circuits

- Resistor Color Bands

| Label | Resistor Color Bands | Computed Resistance |
| --- | --- | --- |
| R1 | Brown, Orange, Brown, Gold | 130 Ω, 5% |
| R2 | Red, Red, Brown, Gold | 220 Ω, 5% |
| R3 | Green, Blue, Brown, Gold | 560 Ω, 5% |
| R4 | Brown, Red, Red, Gold | 1200 Ω, 5% |

- Series Circuits

- Since the circuit is in series, Reff=R1+R2+…+Rn

Reff=R1+R2+R3=130 Ω+ 220 Ω+ 560 Ω= 910 Ω

- In the previous item, the resistance is given by Reff= 910 Ω and voltage is 5 V.

For V1:

V1= R1ReffV= 130 Ω 910 Ω 5V=0.7142857143 V ≈0.714 V or 714.286 mV

For V2:

V2= R2ReffV= 220 Ω 910 Ω 5V=1.208791209 V ≈1.209 V

For V3:

V3= R3ReffV= 560 Ω 910 Ω 5V=3.076923077 V≈3.077 V

- To compute for the theoretical current through the circuit, use Ohm’s Law:

I = VR= 5 V910 Ω=5.94505495 × 10-3A ≈5.495 mA

- Parallel Circuits

- Since the circuit is in parallel, 1Reff=1R1+1R2+…+1Rn

1Reff=1R1+1R2+1R3=1130 Ω+1220 Ω+1560 Ω= 112380080 Ω

Reff=1Reff-1= 112380080Ω-1=71.30899377 Ω ≈71.309 Ω

- In the previous item, the resistance is given by Reff≈71.309 Ω and voltage is 5V.

To solve the theoretical current of each resistor, compute first for the current through the entire network:

Using Ohm’s Law, I = VR= 5 V71.309 Ω =0.07011738262 A ≈0.07A or 70.117 mA

For i1:

i1= 1R11Reffi= 1130 Ω171.309 Ω0.07A =0.03846153846 A ≈0.0385 A or 38.462 mA

For i2:

i2= 1R21Reffi= 1220 Ω171.309 Ω0.07A =0.02272727273 A ≈0.0227 A or 22.727 mA

For i3:

i3= 1R31Reffi= 1560 Ω171.309 Ω0.07A =8.928571429 × 10-3 A ≈8.929 mA

- To compute for the theoretical voltage through the circuit, use Ohm’s Law:

V=IR=0.07A71.309 Ω =5V