CS 20 Laboratory 2: Application of Kirchhoff’s Laws

- KCL and KVL Exercise

- Enumerate all effective nodes of the circuit; include all components connected to each node.

Node A

Node B

Let us denote the effective nodes in the circuit as follows:

Node D

Node C

Figure 1: Visualization of effective nodes

The components of each node are:

- Node A – 5V voltage source, 100 Ω resistor, and 270 Ω resistor

- Node B – 270 Ω resistor, 470 Ω resistor, 220 Ω resistor, 390 Ω resistor, and 560 Ω resistor

- Node C – 5V voltage source, 100 Ω resistor, 220 Ω resistor, and 180 Ω resistor

- Node D - 180 Ω resistor and 390 Ω resistor

- Fill up the table below. Show relevant equations as well as screenshots from Falstad showing that your calculations are correct:

| Element | Voltage across | Current through |
| --- | --- | --- |
| R100Ω | 5V | 0.05 A or 50mA |
| R180Ω | 0.58459 V or 584.59 mV | 0.0032 A or 3.25 mA |
| R220Ω | 1.85 V | 0.0084 A or 8.41 mA |
| R270Ω | 3.15 V | 0.01166 A or 11.66 mA |
| R390Ω | 1.27 V | 0.0032 A or 3.25 mA |
| R470Ω | 0 V | 0 A |
| R560Ω | 0 V | 0 A |
| Source | 5V | 0.6166 A or 61.66 mA |

Solutions:

In order to obtain the desired quantities, we must simplify the circuit. We can neglect R470Ω and R560Ω already because they do not affect the circuit and have no current.

As shown in figure 1, R180Ω and R390Ω are in series so we can simplify their resistance. Let Rs1 be their simplification given by

Rs1=R180Ω+R390Ω=570 Ω

Furthermore, R220Ω and Rs1 are in parallel so we can further simplify them. Let Rs2 be their simplification given by

Rs2=1Rs2-1=1R220Ω+1Rs1-1= 7912540-1=158.7341772 Ω≈158.73 Ω

Now R270Ω and Rs2 are in series so we can further simplify them. Let Rs3 be their simplification given by

Rs3=R270Ω+ Rs2=428.7341772 Ω ≈428.73 Ω

Finally, R100Ω and Rs3 will be left in parallel with the source. This is the simplified version of the circuit.

- For R100Ω and source

Since it is in parallel with the voltage source, by KVL, the voltage is also 5V. Using Ohm’s Law, I100Ω = V100ΩR100Ω= 5 V100 Ω =0.05 A or 50mA

Furthermore, by KCL, we can add the current of R100Ω and Rs3

I = V100ΩR100Ω+Vs3Rs3=5 V100 Ω +5 V428.73 Ω =0.06166223797 A ≈0.6166 A or 61.66 mA

- For R270Ω

Similarly, Rs3 is in parallel with the voltage source so by KVL, its voltage is also 5V. Since it is the simplification of Rs2 and R270Ω in series, we can use voltage division such that

V270Ω= R270ΩRs3Vs3= 270 Ω428.73 Ω5V=3.148804252 V ≈3.15 V

Using Ohm’s Law,  I270Ω = V270ΩR270Ω= 3.15 V270 Ω =0.01166223797 A

≈0.01166 A or 11.66 mA

- For R220Ω

Rs2 is in series with R270Ω so by KCL, they have the same current. Furthermore, it is also the simplification of R220Ω and Rs1 in parallel, so by KVL, both have the same voltage as Rs2, which can be obtained using Ohm’s Law

V220Ω=Vs2= Is2Rs2=11.66 mA158.73 Ω=1.851195748 V ≈1.85 V

I220Ω = V220ΩR220Ω= 1.85 V220 Ω =0.008414526129 A ≈0.0084 A or 8.41 mA

- For R180Ω and R390Ω

Rs1 is in parallel with R220Ω so by KVL, they have the same voltage. Furthermore, it is also the simplification of R180Ω and R390Ω in series, so by KCL, both have the same current as Rs1, which can be obtained using Ohm’s Law

I180Ω= I390Ω = Is1= Vs1Rs1= 1.85 V570 Ω =0.003247711839 A ≈0.0032 A or 3.25 mA

Since they are in series, they will have different voltages

V180Ω = I180Ω R180Ω =3.25 mA180 Ω=0.5845881311 V ≈0.58459 V or 584.59 mV

V390Ω = I390Ω R390Ω =3.25 mA390 Ω=1.266607617 V ≈1.27 V

- Are there any resistors in which no current passes through? Identify which resistor(s), if any. Why is there no current passing through the said resistor(s)?

Yes, R470Ω and R560Ω does not have any current passing. As shown in figure 1, R470Ω is connected to a single node such that both its terminals are connected to only one node. This is also the case for R560Ω. Since these resistors just loop at a single node, and is neither in parallel nor in series, we can neglect them as they do not affect the circuit.

- Equivalent Resistors exercise

- Show a screenshot of your implementation of the circuit in Falstad.

Figure 2: New Circuit Implemented in Falstad

- Prove using resistor collapsing that your implementation has an equivalent resistance of about 88.7Ω. Include step-by-step equations and screenshots.

Figure 3: Visualization of nodes of new circuit

As shown in the figure above (fig 3), R390Ω and R220Ω  are in series while R270Ω and R470Ω are in parallel. We can get the equivalent of these resistors as Re1and Re2, respectively.

Re1=R390Ω+R220Ω=610 Ω

Re2=1Re2-1=1R270Ω+1R470Ω-1= 376345Ω-1=171.4864865 Ω ≈171.49 Ω

R180Ω and R171.49Ω are in series so we can get their equivalent Re3

Re3=R180Ω+R171.49Ω=351.49 Ω

R610Ω and R351.49Ω are in series so we can get their equivalent Re4

Re4=1Re4-1=1R610Ω+1R351.49Ω-1=222.9950808 Ω≈223 Ω

R223Ω and R560Ω are in series so we can get their equivalent Re5

Re5=R223Ω+R560Ω=783 Ω

Finally, R100Ω and R783Ω are in series so we can get their equivalent Req which is the equivalent resistance of the whole circuit

Req=1Req-1=1R100Ω+1R783Ω-1=88.67497169 Ω≈88.7 Ω

Therefore, the equivalent resistance of the circuit is about 88.7 Ω

Q.E.D.

- From the equations gathered in the previous item, show that the current supplied by the 5V source is about 588.7 A

To get the current supplied by the source voltage, we use Ohm’s law

where Req is the equivalent resistance of the circuit computed from the previous item,

I = VR= VRe6= 5V88.7 Ω≈588.7A